MATERIAL WITH RECYCLED CONTENT SPECIFICATION

NEW YORK CITY DEPARTMENT OF DESIGN AND CONSTRUCTION



07 2100 
FIBERGLASS BUILDING INSULATION

FINAL:  
Feb. 12, 2009
Note:  The following specification criteria have been assembled to promote the procurement of post-consumer/post-industrial recycled content in fiberglass insulation. Additional considerations related to product emissions, product toxicity, or other High Performance building criteria are included where appropriate.

PART 1 - GENERAL

1.01  SUMMARY

A. This section includes fiberglass building insulations which meet specified criteria for:

1. Post- consumer and/or post-industrial recycled content; and

2. Reduced formaldehyde emissions (optional)

1.02 REFERENCES / QUALITY ASSURANCE

(for recycled content and other High Performance building criteria)

A. Recycled content: 
1) EPA Comprehensive Procurement Guidelines, www.epa.gov/cpg
2) ASTM D5359, “Glass Cullet Recovered from Waste for Use in Manufacture of Glass Fiber”

1.03
SUBMISSION REQUIREMENTS

A
Manufacturer’s certification of recycled content per section 2.01 of this specification.

B. Manufacturer’s certification of product compliance with emissions requirements per section 2.01 of this specification (optional).

C. Material Safety Data Sheets.

D. Manufacturer’s policy statement on fiberglass insulation recycling programs.

1.04
DELIVERY AND STORAGE

A. Insulation to be stored per manufacturer’s recommendations for allowable temperature and humidity range. Insulation shall not be allowed to become damp. 

B. Where feasible, fiberglass insulation shall be stored separately from materials which have high short-term emissions. Materials with high short-term emissions include, but are not limited to: adhesives, sealants and glazing compounds (specifically those with petrochemical vehicles or carriers); paint, wood preservatives, and finishes; control and/or expansion joint fillers; hard finishes requiring adhesive installation; gypsum board (with associated finish processes and products); and composite or engineered wood products with formaldehyde binders.

C. Where feasible, exposed fiberglass insulation shall not be stored in occupied spaces, near HVAC diffusers (supply or return), or near fresh air intakes.

PART 2 – PRODUCTS

2.01 MATERIALS

A. Environmental Criteria:

1. Recycled Content: 

Fiberglass insulation shall contain a minimum of 20% (combined) post-industrial/post-consumer recycled content. The percentage of recycled content is based on the weight of the component materials.

2. Emissions:


a. Where feasible, provide fiberglass insulation that does not contain formaldehyde binders.
b. Fiberglass insulation in exposed locations and in ceiling plenums (used for HVAC return) shall be encapsulated with a continuous wrap of polyethylene or similar material.

PART 3 –  EXECUTION

3.01
WASTE MANAGEMENT

A. Identify fiberglass insulation manufacturer’s policy for collection or return of construction scrap, unused material, demolition scrap, and/or packaging material. Where feasible, institute demolition and construction waste separation and recycling to take advantage of manufacturer’s programs.

END OF SECTION 07200







